MEDSYS MedSMS Manual

Overview

Message Delivery - messages can be sent two ways:

- Through a web browser – easy to use, easily accessible interface (no installation necessary).  The web interface could be installed in the hospital network or run directly from the online MedSMS web server (http://medsys.lorma.net/medsms).  The interface could be customized if installed on the hospital network (using process flow #2 below).

- Through the MedSMS program – more advanced features.  Automatic sending of data to doctors, like new admissions, lab results, discharges, professional fees, etc.  The default firewall port 8444 should be open.

Process Flow - 2 ways to use MedSMS:

- Through internet connection

- The internet is used to connect to an existing MedSMS Server, which sends the SMS messages for the hospital

- Can be used to try out MedSMS

- No need to buy “load” or purchase a phone

- Currently only for Globe

- Using dedicated phone for hospital (Sony Ericsson K320i is recommended)

- Still functional even if the internet is down

- Can attached several phones (with data capble supporting AT commands)

- A little bit faster to deliver a message, as compared to #1

Installing as a service

To keep the program running in the background always, install msys.exe as a service.  This are the instructions for Windows XP.

1. Download Windows Resource Kit:

http://www.microsoft.com/downloads/details.aspx?familyid=9d467a69-57ff-4ae7-96ee-b18c4790cffd&displaylang=en

2. Click -> Start | Programs | Windows Resouce Kit Tools | Command Shell

3. Type: (make sure Srvany.exe is in msys folder)

"C:\Program Files\Windows Resource Kits\Tools\Instsrv.exe" msys "C:\medsys\apps\msys\Srvany.exe"

to remove:

"C:\Program Files\Windows Resource Kits\Tools\Instsrv.exe" msys remove

4. click start | Run , then regedit.exe

HKLM\SYSTEM\CurrentControlSet\Services\Msys

Create a "Parameters" key

Open the "Parameters" key

Create an "Application" string value (REG_SZ) within the "Parameters" key

Edit "Application" and enter C:\medsys\apps\msys\msys.exe

5. Click -> Start | Settings | Control Panel | Administrative Tools | Services

Right Click Msys then properties

Undel Log On check "Allow Service to interact with desktop"

Server Setup:

On the MedSMS window, enter 203.131.181.114 in the MedSMS Server URL

Set the “Auto-check” checkbox.

Click on DB Utils, check “Auto-open Menu”

The “Read/Send SMS Window” / “Send Window” should only be open on the server when a phone is connected.

Options Settings

You could enter options by clicking on DB Utils then Options from the MedSMS window.

DBPort: 84444

ServerPassword: 888x

Restart program and click on Apply URL to check if can connect